# Get READY!

GET YOUR PROPERTY READY

Being ready for a wildfire starts with creating and maintaining an adequate defensible space around your residence and by hardening your home by using fire resistant building materials.  Defensible space is the buffer you create by installing and maintaining fire-safe landscaping around your house.  This buffer helps to keep the fire away from your home.  Hardening your home means using construction materials that can help your home withstand flying embers finding weak spots in the construction, which can result in your house catching fire.  It takes the combination of defensible space and the hardening of your home to give your house the best chance of surviving a wildfire.

See below for more information on Defensible Space, Hardening Your Home, and Fire Resistant Landscaping.

CREATE AND MAINTAIN DEFENSIBLE SPACE

Keep your property lean and green to help protect your family and home.

Creating defensible space is essential to improve your home’s chance of surviving a wildfire.  It is the buffer you create between a building and the grass, trees, shrubs, or wildland area surrounding it.  This needed space slows or stops the spread of wildfire and protects your home from catching fire, from either direct flame contact or radiant heat.  Defensible space is also important for the protection of the firefighters defending your home.

Research around home destruction vs. home survival in wildland fires point to embers and low intensity fire up against the structure as the main way majorities of homes ignite during a wildland fire.  Embers are burning pieces of wood and/or vegetation that can be carried more than a mile through the wind, which can cause spot fires and ignite homes, debris, and other objects.

Experiments, models, and post-fire studies have shown homes ignite due to the condition of the home and everything around it up to 100 feet from the foundation.  This area is the Home Ignition Zone (HIZ).  In the City of Newport Beach, three zones make up the required 100 feet of HIZ.  There are methods for homeowners to prepare their homes to withstand ember attacks and minimize the likelihood of flames or surface fire touching the home or any attachments.

Zone 1- Home Ignition Zone- Immediate (0-5 feet from structure)

Organic Mulch and Wood Chips

- Organic mulch and wood chip application is prohibited within two feet (2’) of the structure.

      - Must use non-combustible materials such as gravel and decomposed granite within two feet (2’) of the structure.

      - Organic mulch and wood chip application is allowed to be used from three to five feet (3’-5’).

      - The maximum depth of mulch and wood chips in this zone shall be four inches (4”).

    - Use a mosaic (non-uniform) pattern with a maximum application area of 20 feet x 20 feet and a minimum of five feet (5’) separation.

Ground Cover

- Ground cover and grasses shall be maintained less than six inches (6”) in height.

Bushes and Shrubs

- Bushes and shrubs in this zone shall not exceed eighteen inches (18”).

- Bushes and shrubs not under a tree canopy can remain if horizontal and vertical clearances are properly maintained.

- Bushes and shrubs shall not be a continuous bed of vegetation.

      - Bushes and shrubs in this zone shall not exceed eighteen inches (18”) in height.

      - Bushes and shrubs shall have clear horizontal and vertical separation between each other and other vegetation.

      - Existing bushes and shrubs shall be individually spaced or clustered in groups of three (3) specimens or less.

      - Bushes and shrubs, either individually spaced or in a group or cluster, shall not exceed ten feet (10’) in diameter.

      - Bushes and shrubs shall have horizontal separation of three times the height.

Trees

- No trees canopy shall be located within five feet (5’) of the home or structure.

- No trees canopy shall be located within ten feet (10’) of a chimney.

- The Newport Beach Fire Department strongly recommends no vegetation within five feet (5’) of the structure.

Zone 2- Home Ignition Zone- Intermediate (5-30 feet from structure)

Organic Mulch and Wood Chips

- Organic mulch and wood chip application shall be limited to non-continuous use of organic mulch and or wood chips not to exceed four inches (4”) in depth.

      - Use a mosaic (non-uniform) pattern with a maximum application area of 20 feet x 20 feet and a minimum of five feet (5’) separation.

Ground Cover

- Ground cover and grasses shall be maintained less than eighteen inches (18”) in height.

Bushes and Shrubs

- Bushes and shrubs in this zone shall not exceed eighteen inches (18”).

- Bushes and shrubs not under a tree canopy can remain, if vertical and horizontal clearances are properly maintained.

- Bushes and shrubs shall not be a continuous bed of vegetation.

      - Bushes and shrubs in this zone shall not exceed eighteen inches (18”) in height.

      - Bushes and shrubs shall have clear horizontal and vertical separation between each other and other vegetation.

      - Existing bushes and shrubs shall be individually spaced or clustered in groups of three (3) specimens or less.

      - Bushes and shrubs, either individually spaced or in a group or cluster, shall not exceed ten feet (10’) in diameter.

      - Bushes and shrubs shall have horizontal separation of three times the height.

      - Top of bush or shrub shall have six feet (6’) of clear separation from the lowest part of the tree canopy above.

- Bushes and shrubs up to eighteen inches (18”) are allowed under an existing tree canopy in this zone. Minimum clear space under a tree canopy is five feet (5’).

- Bushes and shrubs that do not provide minimum vertical or horizontal separation shall be removed.

Trees

- Trees shall be horizontally spaced three times the size of the mature canopy.

- Trees shall be maintained (pruned, laced out, and limbed up) to provide vertical and horizontal separation between other fuels.

Zone 3- Home Ignition Zone- Extended (30-100 feet from structure)

Organic Mulch and Wood Chips

- Organic mulch and wood chip application is allowed.

- No limitations on application areas.

Bushes and Shrubs

- Bushes and shrubs not under a tree canopy can remain, if properly maintained.

      - Bushes and shrubs in excess of two feet (2’) shall not be under a tree canopy.

- Bushes and shrubs shall not be a continuous bed of vegetation.

      - Bushes and shrubs in this zone shall not exceed two feet (2’) in height.

      - Bushes and shrubs shall have clear horizontal and vertical separation between each other and other vegetation.

      - Existing bushes and shrubs shall be individually spaced or clustered in groups of three (3) specimens or less.

      - Bushes and shrubs, either individually spaced or in a group or cluster, shall not exceed ten feet (10’) in diameter.

      - Bushes and shrubs shall have horizontal separation of three times the height.

      - Top of bush or shrub shall have six feet (6’) of clear separation from the lowest part of the tree canopy above.

- Bushes and shrubs up to two feet (2’) are allowed under an existing tree canopy. Minimum clear space under a tree canopy is six feet (6’)

- Bushes and shrubs that do not provide minimum vertical separation shall be removed.

Trees

- Trees shall be horizontally spaced three times the size of the mature canopy

Trees shall be maintained (pruned, laced out, and limbed up) to provide vertical and horizontal separation between other fuels.

HARDEN YOUR HOME

Flying embers can destroy homes up to a mile from a wildfire.  Harden your home now before a fire starts by using ember-resistant building materials.

Here are some things you can do to harden your home and make it more fire resistant.

Roof

The roof is the most vulnerable part of your home.  Homes with wood or shingle roofs are at high risk of being destroyed during a wildfire.  Build your roof or re-roof with materials such as composition, metal or tile.  Block any spaces between roof decking and covering to prevent embers from catching.

Vents

Vents on homes create openings for flying embers.

Cover all vent openings with 1/8-inch to 1/4-inch metal mesh.  Do not use fiberglass or plastic mesh because they can melt and burn.

Protect vents in eaves or cornices with baffles to block embers (mesh is not enough).

Eaves and Soffits

Eaves and soffits should be protected with ignition-resistant* or non-combustible materials.

Windows

Heat from a wildfire can cause windows to break even before the home ignites. This allows burning embers to enter and start fires inside. Single-paned and large windows are particularly vulnerable.

Install dual-paned windows with one pane of tempered glass to reduce the chance of breakage in a fire.

Consider limiting the size and number of windows that face large areas of vegetation.

Walls

Wood products, such as boards, panels or shingles, are common siding materials. However, they are combustible and not good choices for fire-prone areas.

Build or remodel your walls with ignition resistant* building materials, such as stucco, fiber cement, wall siding, fire retardant, treated wood, or other approved materials.

Be sure to extend materials from the foundation to the roof.

Decks

Surfaces within 10 feet of the building should be built with ignition-resistant*, non-combustible, or other approved materials.

Ensure that all combustible items are removed from underneath your deck.

Rain Gutters

Screen or enclose rain gutters to prevent accumulation of plant debris.

Patio Cover

Use the same ignition-resistant* materials for patio coverings as a roof.

Chimney

Cover your chimney and stovepipe outlets with a non-combustible screen. Use metal screen material with openings no smaller than 3/8-inch and no larger than 1/2-inch to prevent embers from escaping and igniting a fire.

Garage

Have a fire extinguisher and tools such as a shovel, rake, bucket, and hoe available for fire emergencies.

Install weather stripping around and under the garage door to prevent embers from blowing in.

Store all combustible and flammable liquids away from ignition sources.

Fences

Consider using ignition-resistant* or non-combustible fence materials to protect your home during a wildfire.

Driveways and Access Roads

Driveways should be built and maintained in accordance with state and local codes to allow fire and emergency vehicles to reach your home.  Consider maintaining access roads with a minimum of 10 feet of clearance on either side, allowing for two-way traffic.

Ensure that all gates open inward and are wide enough to accommodate emergency equipment.

Trim trees and shrubs overhanging the road to allow emergency vehicles to pass.

Address
Make sure your address is clearly visible from the road.

Water Supply

Consider having multiple garden hoses that are long enough to reach all areas of your home and other structures on your property. If you have a pool or well, consider getting a pump.

*Ignition-resistant building materials are those that resist ignition or sustained burning when exposed to embers and small flames from wildfires. Examples of ignition-resistant materials include “non-combustible materials” that do not burn, exterior grade fire-retardant-treated wood lumber, fire-retardant-treated wood shakes and shingles listed by the State Fire Marshal (SFM) and any material that has been tested in accordance with SFM Standard 12-7A-5.

INSTALL AND MAINTAIN FIRE-SAFE LANDSCAPING

Fire-safe landscaping is not necessarily the same thing as a well-maintained yard.  Fire-safe landscaping uses fire-resistant plants strategically planted to resist the spread of fire to your home.

The good news is that you do not need to spend a lot of money to make your landscape fire-safe and fire-safe landscaping can increase your property value and conserve water while beautifying your home.

Choose Fire-Resistant Plants and Materials

- Create fire-safe zones with stone walls, patios, decks, and roadways.

- Use rock, mulch, flowerbeds, and gardens as ground cover for bare spaces and as effective firebreaks.

- There are no “fire-proof” plants.  Select high-moisture plants that grow close to the ground and have a low sap or resin content.

- Choose fire-retardant plant species that resist ignition such as rockrose, ice plant, and aloe.

- Select fire-resistant shrubs such as hedging roses, bush honeysuckles, currant, cotoneaster, sumac, and shrub apples.

- Plant hardwood, maple, poplar, and cherry trees that are less flammable than pine, fir, and other conifers.